- PURPOSE:  Establish High Capacity Line of Site (HCLOS) link between JNN #### and CPN ##### to allow for increased speed and bandwidth.

- EXECUTION DTG:  DATE/TIME local time

- ASSEMBLAGES AFFECTED:

  - JNN ####

  - CPN #####

- UNIT RESPONSIBILITIES:

  - BSTB:

    - Complete the following tasks with HLCOS ####

      - HCLOS #### will install HCLOS band 3 antenna at 11S NV 12345678.  Azimuth 180 degrees, polarization HORIZONTAL.

      - Receive Frequency:   1345.0

      - Transmit Frequency:  1453.0

      - Link antenna will be connected to fa0/3.

      - Radio IP Address:  192.168.0.2 255.255.255.248

      - Connect radio IP cable to port three of shelter switch

      - Configure shelter switch with following configurations:

Interface fa0/3

Switchport mode access

Switchport access vlan 103

Interface fa0/24

Switchport trunk allow vlan add 103

    - Configure JNN #### NT2R with the following configurations:

Interface G0/0.103

Ip address 192.168.0.1 255.255.255.248

Encapsulation dot1q 103

Interface Tunnel 103

Ip unnumbered loopback 0

Tunnel source g0/0.103

Tunnel destination 192.168.0.4

  - 1-2 CAV:

    - Complete the following tasks with HLCOS ####

      - HCLOS ##### will install HCLOS band 3 antenna at 11S NV 987654.  Azimuth 360 degrees, polarization HORIZONTAL.

      - Receive Frequency:   1453.0

      - Transmit Frequency:  1345.0

      - Radio IP Address:  192.168.0.3 255.255.255.248

      - Connect radio IP cable to port g0/40 on CPN NT2S

      - Configure shelter switch with following configurations:

    - Complete the following configurations for CPN ##### NT2S:

Interface G0/40

Switchport mode access

Switchport access vlan 103

    - Complete the following configurations for CPN 77673 NT2R:

Interface G0/0.103

Ip address 192.168.0.4 255.255.255.248

Encapsulation dot1q 103

Interface Tunnel 103

Ip unnumbered loopback 0

Tunnel source g0/0.103

Tunnel destination 192.168.0.1

- COORDINATING INSTRUCTIONS:

  - JNN #### is the master of this link and will direct all installation and troubleshooting procedures.

  - NETOPS will be notified immediately of any problems that occur which may result in this link not being established.

- COMSEC REQUIREMENTS:

  - #-# CAV:  Draw the current KIV key prior to execution date and continue to draw the monthly update while this shot is active.

- POCs:

  - NETOPS:

  - BDE S6:

  - JNN ####:

  - CPN #####:

WRITTEN BY:

APPROVED BY:

//ORIGINAL SIGNED//

BDE S6 NAME

MAJ, SC

ATTACHMENTS:

1. None

DISTRIBUTION:

- NETOPS

- CDR, B, BSTB

- JNN ####

- S6, #####